INTERFACAGE DES MICROCONTROLEURS

Les interfaces des microcontrôleurs permettent la communication entre le microcontrôleur et les périphériques via des bus. Cette interconnexion implique des aspects matériels (connexions, signaux) et logiciels (protocoles, gestion des données).

Les interfaces courantes incluent SPI, I2C, UART et GPIO. Une bonne conception d'interface assure une transmission fiable et efficace des données dans les systèmes embarqués.

Figure 1: Les différents types d’interfaces

| Analog Inputs/Outputs Advantages Disadvantages - Simple interface • High cost for higher - Low cost for low- resolutions resolutions • Not all microcontrollers - High speed have analog I/O built-in - Low programming • Complicates the circuit overhead design when external ADC or DAC are needed. - Short distance, few meters maximum. |  |
| --- | --- |
|  | 3 |

| Analog Inputs/Outputs Voltage type: Typical ranges Current type: Typical ranges • 0 to 2.5V • 0-20mA • 0 to 4V • 4-20mA - 0 to 5V - 0 to 10V • +/- 2.5V • +/- 4V • +/- 5V |  |
| --- | --- |
|  | 4 |

| I2C - The I2C bus was designed by Philips in the early ’80s to allow easy communication between components which reside on the same circuit board. Philips Semiconductors migrated to NXP in 2006. - The name I2C translates into “Inter IC”. Sometimes the bus is called IIC or I²C bus. - Typical application to connect: - Some intelligent control, usually a single-chip microcontroller - General-purpose circuits like LCD and LED drivers, remote I/O ports, RAM, EEPROM, real-time clocks or A/D and D/A converters - Application-oriented circuits such as digital tuning and signal processing circuits for radio and video systems, temperature sensors, and smart cards |  |
| --- | --- |
|  | 5 |

| I2C – Most significant features - Only two bus lines are required; a serial data line (SDA) and a serial clock line (SCL). - Each device connected to the bus is software addressable by a unique address and simple master/slave relationships exist at all times; masters can operate as master- transmitters or as master-receivers. - It is a true multi-master bus including collision detection and arbitration to prevent data corruption if two or more masters simultaneously initiate data transfer. - Serial, 8-bit oriented, bidirectional data transfers can be made at up to 100 kbit/s in the Standard-mode, up to 400 kbit/s in the Fast-mode, up to 1 Mbit/s in Fast-mode Plus, or up to 3.4 Mbit/s in the High-speed mode. - Serial, 8-bit oriented, unidirectional data transfers up to 5 Mbit/s in Ultra Fast- mode - On-chip filtering rejects spikes on the bus data line to preserve data integrity. - The number of ICs that can be connected to the same bus is limited only by a maximum bus capacitance. More capacitance may be allowed under some conditions. |  |
| --- | --- |
|  | 6 |

| VCC | I2C supply voltage, typically ranging from 1.2 V to 5.5 V |
| --- | --- |
| GND | Common ground |
| SDA | Serial data (I2C data line) |
| SCL | Serial clock (I2C clock line) |
| Rp | Pull-up resistance (a.k.a. I2C termination) |
| Rs | Serial resistance |
| Cp | Wire capacitance |
| Cc | Cross channel capacitance |

| I2C – Typical Setup 7 |  |
| --- | --- |

| Example of I2C-bus applications |  |
| --- | --- |
|  | 8 |

| Timing - Data transfer is initiated with a start bit (S) signaled by SDA being pulled low while SCL stays high. - SCL is pulled low, and SDA sets the first data bit level while keeping SCL low (during blue bar time). - The data are sampled (received) when SCL rises for the first bit (B1). For a bit to be valid, SDA must not change between a rising edge of SCL and the subsequent falling edge (the entire green bar time). - This process repeats, SDA transitioning while SCL is low, and the data being read while SCL is high (B2, ...Bn). - The final bit is followed by a clock pulse, during which SDA is pulled low in preparation for the stop bit. - A stop bit (P) is signaled when SCL rises, followed by SDA rising. |  |
| --- | --- |
|  | 9 |

| Limitations - The assignment of slave addresses is one weakness of I²C. Seven bits is too few to prevent address collisions between the many thousands of available devices. - 10-bit I²C addresses are not yet widely used - Automatic bus configuration is a related issue. A given address may be used by a number of different protocol-incompatible devices in various systems, and hardly any device types can be detected at runtime. For example, 0x51 may be used by a 24LC02 or 24C32 EEPROM EEPROM, with incompatible addressing; or by a PCF8563 RTC, which cannot reliably be distinguished from - I²C supports a limited range of speeds. - Devices are allowed to stretch clock cycles to suit their particular needs, which can starve bandwidth needed by faster devices and increase latencies when talking to other device addresses. - Because I²C is a shared bus, there is the potential for any device to have a fault and hang the entire bus. For example, if any device holds the SDA or SCL line low, it prevents the master from sending START or STOP commands to reset the bus. Thus it is common for designs to include a reset signal that provides an external method of resetting the bus devices. - Because of these limits (address management, bus configuration, potential faults, speed), few I²C bus segments have even a dozen devices. It is common for systems to have several such segments. |  |
| --- | --- |
|  | 10 |

| Further reading - https://www.i2c-bus.org/ - https://www.nxp.com/docs/en/user-guide/UM10204.pdf - https://en.wikipedia.org/wiki/I%C2%B2C |  |
| --- | --- |
|  | 11 |

| Serial Peripheral Interface Bus - The Serial Peripheral Interface bus (SPI) is a synchronous serial communication interface specification used for short distance communication, primarily in embedded systems. The interface was developed by Motorola in the late 1980s and has become a de facto standard. Typical applications include Secure Digital cards and liquid crystal displays. - SPI devices communicate in full duplex mode using a master-slave architecture with a single master. The master device originates the frame for reading and writing. Multiple slave devices are supported through selection with individual slave select (SS) lines. - Sometimes SPI is called a four-wire serial bus, contrasting with three-, two-, and one-wire serial buses. The SPI may be accurately described as a synchronous serial interface, but it is different from the Synchronous Serial Interface (SSI) protocol, which is also a four-wire synchronous serial communication protocol. SSI Protocol employs differential signaling and provides only a single simplex communication channel. |  |
| --- | --- |
|  | 12 |

| The SPI bus specifies four logic signals: - SCLK: Serial Clock (output from master). - MOSI: Master Output Slave Input, or Master Out Slave In (data output from master). - MISO: Master Input Slave Output, or Master In Slave Out (data output from slave). - SS: Slave Select (often active low, output from master). |  |
| --- | --- |
|  | 13 |

| Independent/daisy chain configuration master and three independent slaves master and cooperative slaves |  |
| --- | --- |
|  | 14 |

| SPI – Advantages - Full duplex communication in the default version of this protocol. - Push-pull drivers (as opposed to open drain) provide good signal integrity and high speed - Higher throughput than I²C or SMBus. Not limited to any maximum clock speed, enabling potentially high speed - Complete protocol flexibility for the bits transferred - Not limited to 8-bit words - Arbitrary choice of message size, content, and purpose - Extremely simple hardware interfacing - Typically lower power requirements than I²C or SMBus due to less circuitry (including pull up resistors) - No arbitration or associated failure modes - Slaves use the master's clock and do not need precision oscillators - Slaves do not need a unique address — unlike I²C or GPIB or SCSI - Transceivers are not needed - Uses only four pins on IC packages, and wires in board layouts or connectors, much fewer than parallel interfaces - At most one unique bus signal per device (chip select); all others are shared - Signals are unidirectional allowing for easy Galvanic isolation - Simple software implementation |  |
| --- | --- |
|  | 15 |

| SPI – Disadvantages - Requires more pins on IC packages than I²C, even in the three-wire variant - No in-band addressing; out-of-band chip select signals are required on shared buses - No hardware flow control by the slave (but the master can delay the next clock edge to slow the transfer rate) - No hardware slave acknowledgment (the master could be transmitting to nowhere and not know it) - Typically supports only one master device (depends on device's hardware implementation) - No error-checking protocol is defined - Without a formal standard, validating conformance is not possible - Only handles short distances compared to RS-232, RS-485, or CAN-bus. (can be extended with use of transceivers like RS-422) - Many existing variations, making it difficult to find development tools like host adapters that support those variations - SPI does not support hot swapping (dynamically adding nodes). - Interrupts must either be implemented with out-of-band signals or be faked by using periodic polling similarly to USB 1.1 and 2.0 - Some variants like Multi I/O SPI and three-wire serial buses are half-duplex. |  |
| --- | --- |
|  | 16 |

| SPI – Applications - Sensors: temperature, pressure, ADC, touchscreens, video game controllers - Control devices: audio codecs, digital potentiometers, DAC - Camera lenses: Canon EF lens mount - Communications: Ethernet, USB, USART, CAN, IEEE 802.15.4, IEEE 802.11, handheld video games - Memory: flash and EEPROM - Real-time clocks - LCD, sometimes even for managing image data - Any MMC or SD card |  |
| --- | --- |
|  | 17 |

| - Wire® - 1-Wire is a device communications bus system designed by Dallas Semiconductor Corp. that provides low-speed data, signaling, and power over a single conductor. - 1-Wire is similar in concept to I²C, but with lower data rates and longer range. It is typically used to communicate with small inexpensive devices such as digital thermometers and weather instruments. A network of 1-Wire devices with an associated master device is called a MicroLAN. - One distinctive feature of the bus is the possibility of using only two wires: data and ground. To accomplish this, 1-Wire devices include an 800 pF capacitor to store charge, and to power the device during periods when the data line is active. |  |
| --- | --- |
|  | 18 |

| 1-Wire® |  |
| --- | --- |
|  | 19 |

| - Wire® – Communication - In any MicroLan, there is always one master in overall charge. The master initiates activity on the bus, simplifying the avoidance of collisions on the bus. Protocols are built into the software to detect collisions. After a collision, the master retries the required communication. - Many devices can share the same bus. Each device on the bus has a unique 64-bit serial number. The least significant byte of the serial number is an 8-bit number that tells the type of the device. The most significant byte is a standard (for the 1-wire bus) 8-bit CRC. - There are several standard broadcast commands, as well as commands used to address a particular device. The master can send a selection command, then the address of a particular device. The next command is executed only by the addressed device. |  |
| --- | --- |
|  | 20 |

| 1-Wire® – Example |  |
| --- | --- |
|  | 21 |

| - Wire® – Example - Unique 1-Wire interface requires only one port pin for communication - Multidrop capability simplifies distributed temperature sensing applications - Requires no external components - Can be powered from data line. Power supply range is 3.0V to 5.5V - Zero standby power required - Measures temperatures from -55°C to +125°C. Fahrenheit equivalent is -67°F to +257°F - ±0.5°C accuracy from -10°C to +85°C - Thermometer resolution is programmable from 9 to 12 bits - Converts 12-bit temperature to digital word in 750 ms (max.) - User-definable, nonvolatile temperature alarm settings - Alarm search command identifies and addresses devices whose temperature is outside of programmed limits (temperature alarm condition) - Applications include thermostatic controls, industrial systems, consumer products, thermometers, or any thermally sensitive system |  |
| --- | --- |
|  | 22 |

| - Wire® – Example - Apple MagSafe and MagSafe2 connector-equipped power supplies, displays, and Mac laptops use the 1-Wire protocol to send and receive data to and from the connected Mac laptop, via the middle pin of the connector. Data include power supply model, wattage, and serial number; and laptop commands to send full power, and illuminate the connector LEDs red or green. - Genuine Dell laptop power supplies use the 1-Wire protocol to send data via the third wire to the laptop (about power, current and voltage ratings). The laptop will then refuse charging if the adapter does not meet requirements. |  |
| --- | --- |
|  | 23 |

| PWM output - A Pulse Width Modulation (PWM) Signal is a method for generating an analog signal using a digital source. - A PWM signal consists of two main components that define its behavior: - duty cycle – the amount of time the signal is in a high (on) state as a percentage of the total time of it takes to complete one cycle - frequency – determines how fast the PWM completes a cycle. - By cycling a digital signal off and on at a fast enough rate, and with a certain duty cycle, the output will appear to behave like a constant voltage analog signal when providing power to devices. |  |
| --- | --- |
|  | 24 |

| PWM output 25% duty cycle 50% duty cycle 75% duty cycle Frequency: - Heating elements or systems with slow response times: 10-100 Hz or higher - DC electric motors: 5-10 kHz or higher - Power supplies or audio amplifiers: 20-200 kHz or higher - Certain systems may need faster frequencies depending on the type of response desired. |  |
| --- | --- |
|  | 25 |

| 82C55 – UNIVERSAL PARALLEL PORT |  |
| --- | --- |
|  | 26 |

| About 82C55 - The 82C55 is a popular interfacing component, that can interface any TTL-compatible I/O device to a microprocessor. - It is used to interface to the keyboard and a parallel printer port in PCs (usually as part of an integrated chipset). - Requires insertion of wait states if used with a microprocessor using higher that an 8 MHz clock. - PPI has 24 pins for I/O that are programmable in groups of 12 pins and has three distinct modes of operation. |  |
| --- | --- |
|  | 27 |

| 82C55 : Pin Layout |  |
| --- | --- |
|  | 28 |

| Basic Mode Definitions and Bus Int - Mode 0 - Basic I/O - Mode 1 - Strobe I/O - Mode 2 - Bi-Dir Bus 29 |  |
| --- | --- |

| Programming 82C55 |  |
| --- | --- |
|  | 30 |

| Mode 0 (Basic Input/Output). - This functional configuration provides simple input and output operations for each of the three ports. - No “handshaking” is required, data is simply written to or read from a specified port. |  |
| --- | --- |
|  | 31 |

| Mode 0 Port definition |  |
| --- | --- |
|  | 32 |

| 82C55: Mode 0, Scan Display |  |
| --- | --- |
|  | 33 |

| 82C55: Mode 0, Scan Display - Mode 0 operation causes the 82C55 to function as a buffered input device or as a latched output device. - In previous example, both ports A and B are programmed as (mode 0) simple latched output ports. - Port A provides the segment data inputs to display and port B provides a means of selecting one display position at a time. - Different values are displayed in each digit via fast time multiplexing. |  |
| --- | --- |
|  | 34 |

| 82C55: Mode 0, Scan Key |  |
| --- | --- |
|  | 35 |

| 82C55: Mode 0 Operation |  |
| --- | --- |
|  | 36 |

| MODE 1 (Strobed Input/Output) - This functional configuration provides a means for transferring I/O data to or from a specified port in conjunction with strobes or “handshaking” signals. - In mode 1, Port A and Port B use the lines on Port C to generate or accept these “handshaking” signals |  |
| --- | --- |
|  | 37 |

| Mode 1 Basic functional Definitions - Two Groups (Group A and Group B). - Each group contains one 8-bit data port and one 4-bit control/data port. - The 8-bit data port can be either input or output Both inputs and outputs are latched. - The 4-bit port is used for control and status of the 8-bit data port. |  |
| --- | --- |
|  | 38 |

| 82C55: Mode 1 Strobed Input - ~STB : The strobe input loads data into the port latch on a 0- to-1 transition. - IBF : Input buffer full is an output indicating that the input latch contain information. - INTR : Interrupt request is an output that requests an interrupts. - INTE : The interrupt enable signal is neither an input nor an output; it is an internal bit programmed via the PC4 (port A) or PC2 (port B) bits. - PC7,PC6 : The port C pins 7 and 6 are general purpose I/O pings that are available for any purpose. |  |
| --- | --- |
|  | 39 |

| 82C55: Mode 1 Strobed Input Signal definitions for Mode 1 Strobe Input |  |
| --- | --- |
|  | 40 |

| 82C55: Mode 1 Input Exam. - Keyboard encoder debounces the key-switches, and provides a strobe whenever a key is depressed. - DAV is activated on a key press strobing the ASCII-coded key code into Port A. 41 |  |
| --- | --- |

| 82C55 : Mode 1 Output Exam. - ~OBF : Output buffer full is an output that goes low when data is latched in either port A or port B. Goes low on ~ACK. - ~ACK : The acknowledge signal causes the ~OBF pin return to 0. This is a response from an external device. - INTR : Interrupt request is an output that requests an interrupt. - INTE : The interrupt enable signal is neither an input nor an output; it is an internal bit programmed via the PC6(Port A) or PC2(port B) bits. - PC5,PC4 : The port C pins 5 and 4 are general-purpose I/O pins that are available for any purpose. |  |
| --- | --- |
|  | 42 |

| 82C55 : Mode 1 Output Exam. |  |
| --- | --- |
|  | 43 |

| 82C55: Mode 2 Bi-directional Operation - This functional configuration provides a means for communicating with a peripheral device or structure on a single 8-bit bus for both transmitting and receiving data (bidirectional bus I/O). - “Handshaking” signals are provided to maintain proper bus flow discipline in a similar manner to MODE 1. - Interrupt generation and enable/disable functions are also available. |  |
| --- | --- |
|  | 44 |

| MODE 2 Basic Functional Definitions: - Used in Group A only. - One 8-bit, bi-directional bus port (Port A) and a 5-bit control port (Port C). - Both inputs and outputs are latched. - The 5-bit control port (Port C) is used for control and status for the 8-bit, bi-directional bus port (Port A). |  |
| --- | --- |
|  | 45 |

| 82C55: Mode 2 Bi-directional Operation - INTR : Interrupt request is an output that requests an interrupt. - ~OBF : Output Buffer Full is an output indicating that that output buffer contains data for the bi-directional bus. - ~ACK : Acknowledge is an input that enables tri-state buffers which are otherwise in their high-impedance state. - ~STB : The strobe input loads data into the port A latch. - IBF : Input buffer full is an output indicating that the input latch contains information for the external bi-directional bus. - INTE : Interrupt enable are internal bits that enable the INTR pin. BIT PC6(INTE1) and PC4(INTE2). - PC2,PC1,PC0 : These port C pins are general-purpose I/O pins that are available for any purpose. |  |
| --- | --- |
|  | 46 |

| 82C55: Mode 2 Bi-directional Operation - Timing diagram is a combination of the Mode 1 Strobed Input and Mode 1 Strobed Output Timing diagrams. |  |
| --- | --- |
|  | 47 |

| Mode 2 Timing Diagram |  |
| --- | --- |
|  | 48 |

| Mode definition summary |  |
| --- | --- |
|  | 49 |

|  | 50 |
| --- | --- |

| 8253 Programmable interval timer |  |
| --- | --- |
|  | 51 |

| Agenda - Basic Description of the 8253 - Pin Configuration of the 8253 - Block Diagram of the 8253 - System Interfacing of the 8253 - Interfacing the 8253 to the 8086 Processor - Programming the 8253 - Operating Modes of the 8253 |  |
| --- | --- |
|  | 52 |

| Basic Description of the 8253 - The 8253 is programmable interval timer/counter specifically designed for use with the Intel microcomputer systems. - The 8253 solves one of the most common problems in any microcomputer system, the generation of accurate time delays under software control. - Instead of setting up timing loops in systems software, the programmer configures the 8253 to match his requirements, initializes one of the counters of the 8253 with the desired quantity, then upon command the 8253 will count out the delay and interrupt the CPU when it has completed its tasks. - It is easy to see that the software overhead is minimal and that multiple delays can easily be maintained by assignment of priority levels. |  |
| --- | --- |
|  | 53 |

| Pin Configuration of the 8253 - The pin configuration and pin description of the 8253 are shown in Figure 1 and Table 1 respectively. - D7-0 (Bidirectional Data Bus): - Three-state 8-bit bidirectional data bus used when writing control words and count values, and reading count values upon reception of WR’ and RD’ signals from CPU. - CS’ (Chip Select): - Data transfer with the CPU is enabled when this pin is at low level. - When at high level, the data bus (D0 thru D7) is switched to high impedance state where neither writing nor reading can be executed. - Internal registers, however, remain unchanged. |  |
| --- | --- |
|  | 54 |

| Pin Configuration of the 8253 - RD’ (Read Input): - Data can be transferred from the 8253 to CPU when this pin is at low level. - WR’ (Write Input): - Data can be transferred from CPU to 8253 when this pin is at low level. - A1-0 (Address Input): - One of the three internal counters or the control word register is selected by A0/A1 combination. - These two pins are normally connected to the two lower order bits of the address bus. |  |
| --- | --- |
|  | 55 |

| Pin Configuration of the 8253 - CLK0-2 (Clock Input): - Supply of three clock signals to the three counters incorporated in 8253. - GATE0-2 (Gate Input): - Control of starting, interruption, and restarting of counting in the three respective counters in accordance with the set control word contents. - OUT0-2 (Counter Out): - Output of counter output waveform in accordance with the set mode and count value. |  |
| --- | --- |
|  | 56 |

| Pin configuration of the 8253 . |  |
| --- | --- |
|  | 57 |

| Pin description of the 8253 |  |
| --- | --- |
|  | 58 |

| Block Diagram of the 8253 - The block diagram of the 8253 and the description of the basic operations are shown in Figure 2 and Table 2 respectively. - Data Bus Buffer: - The 3-state, bi-directional, 8-bit buffer is used to interface the 8253 to the system data bus. - Data is transmitted or received by the buffer upon execution of IN or OUT CPU instructions. - The Data Bus Buffer has three basic functions. - (1) Programming the MODES of the 8253. - (2) Loading the count registers. - (3) Reading the count values. |  |
| --- | --- |
|  | 59 |

| Block Diagram of the 8253 - Read/Write Logic: - The Read/Write Logic accepts inputs from the sys- tem bus and in turn generates control signals for overall device operation. - It is enabled or disabled by CS’ so that no operation can occur to change the function unless the device has been selected by the system logic. - Control Word Register: - The Control Word Register is selected when A0A1 = 11. - It then accepts information from the data bus buffer and stores it in a register. - The information stored in this register controls the operation MODE of each counter, selection of binary or BCD counting and the loading of each count register. - The Control Word Register can only be written into; no read operation of its contents is available. |  |
| --- | --- |
|  | 60 |

| Block Diagram of the 8253 - Counter 0, Counter 1, Counter # 2: - These three functional blocks are identical in operation so only a single counter will be described. - Each Counter consists of a single, 16-bit, pre-settable, DOWN counter. - The counter can operate in either binary or BCD and its input, gate and output are configured by the selection of MODES stored in the Control Word Register. - The counters are fully independent and each can have separate MODE configuration and counting operation, binary or BCD. - Also, there are special features in the control word that handle the loading of the count value so that software overhead can be minimized for these functions. - The reading of the contents of each counter is available to the programmer with simple READ operations for event counting applications and special commands and logic are included in the 8253 so that the contents of each counter can be read "on the fly" without having to inhibit the clock input. |  |
| --- | --- |
|  | 61 |

| Block diagram of the 8253 |  |
| --- | --- |
|  | 62 |

| Description of basic operations of the 8253 |  |
| --- | --- |
|  | 63 |

| System Interfacing of the 8253 - The 8253 is a component of the Intel microcomputer systems and interfaces in the same manner as all other peripherals of the family (See Figure 3). - It is treated by the systems software as an array of peripheral I/O ports; three are counters and the fourth is a control register for MODE programming. - Basically, the select inputs A0, A1 connect to the A0, A1 address bus signals of the CPU. - The CS’ can be derived directly from the address bus using a linear select method. - Or it can be connected to the output of a decoder, such as an Intel 8205 for larger systems. |  |
| --- | --- |
|  | 64 |

66

(A1  A0) (D0-D7)

Step(2):  Design control logic (IOW’ & IOR’)

Interfacing the 8253 to the 8086 Processor

- Example 1: Show how to interface the 8253 to the low byte of the 8086 (D0-D7). Assume the following I/O port addresses are used.

Step (1): Design the address decoding

| Figure 3: System Interfacing of the 8253. |  |
| --- | --- |
|  | 65 |

| A15-A12 | A11-A8 | A7-A4 | A3 |  | A2 | A1 | A | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 0000 0000 0000 | 0000 0000 0000 0000 | 0000 0000 0000 0000 | 1 1 1 1 | 0 0 1 1 | 0 1 0 1 |  | 0 0 0 0 |  |
| Chip Select (CS’) |  |  |  |  | Port elect |  |  | Enable Even Byte |

| Figure 4: Circuit interface of the 8253 in Example 1. |  |
| --- | --- |
|  | 67 |

| Programming the 8253 - The complete functional definition of the 8253 is programmed by the systems software. - A set of control words must be sent out by the CPU to initialize each counter of the 8253 with the desired MODE and quantity information. - Prior to initialization, the MODE, count and output of all counters is undefined. - These control words program the MODE, Loading sequence and selection of binary or BCD counting. - Once programmed, the 8253 is ready to perform whatever timing tasks it is assigned to accomplish. |  |
| --- | --- |
|  | 68 |

| Programming the 8253 - All of the MODES for each counter are programmed by the systems software by simple I/O operations. - Each counter of the 8253 is individually programmed by writing a control word into the Control Word Register (A1A0 = 11). - A program intending to use the 8253 must provide the following sequence of actions: - (1) Setting the control word - (2) Setting the count value |  |
| --- | --- |
|  | 69 |

| Programming the 8253 - The format of the Control Word is shown in Figure 5. - SC1-SC0 (Select Counter): Select the desired counter as shown in Table 3. - RL1-RL0 (Read/Load): Set the reading/loading format of the initial count value as shown in Table 4. - M2-M1-M0 (Modes): Specify the operation mode of the 8253 as shown in Table 5. - BCD (Count Mode): Specify the operation count mode BCD/binary as shown in Table 6. |  |
| --- | --- |
|  | 70 |

| Figure 5: Format of the Control Word of the 8253. |  |
| --- | --- |
|  | 71 |

| Table 3: Selection of set counter in the 8253. Table 4: Count value reading/loading format setting in the 8253. |  |
| --- | --- |
|  | 72 |

| Table 5: Operation waveform mode setting in the 8253. Table 6: Operation count setting in the 8253. |  |
| --- | --- |
|  | 73 |

| Programming the 8253 Setting the Count Value - Example 2: Program the 8253 shown in the next figure according to the following settings: - Counter #0: Read/Load LSB only, Mode 3, Binary count, count valu 3H - Counter #1: Read/Load MSB only, Mode 5, Binary count, count value AA00H - Counter #2: Read/Load LSB and MSB, Mode 0, BCD count, count value 1234 | e |
| --- | --- |
|  | 74 |

| Figure 6: Circuit interface of Example 2. |  |
| --- | --- |
|  | 75 |

| MOV | AL, 1EH |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OUT | 0EH, AL | ; | Counter | #0 | control word setting |  |
| MOV | AL, 6AH |  |  |  |  |  |
| OUT | 0EH, AL | ; | Counter | #1 | control word setting |  |
| MOV | AL, 0B1H |  |  |  |  |  |
| OUT | 0EH, AL | ; | Counter | #2 | control word setting |  |
| MOV | AL, 03H |  |  |  |  |  |
| OUT | 08H, AL | ; | Counter | #0 | control value setting |  |
| MOV | AL, 0AAH |  |  |  |  |  |
| OUT | 0AH, AL | ; | Counter | #1 | control value setting |  |
| MOV | AL, 34H |  |  |  |  |  |
| OUT | 0CH, AL | ; | Counter | #2 | count value setting (LSB) |  |
| MOV | AL, 12H |  |  |  |  |  |
| OUT | 0CH, AL | ; | Counter | #2 | count value setting (MSB) |  |
|  |  |  |  |  |  | 76 |

| Operation Modes of the 8253 - Each one of the three counters can be configured to operate in one of six modes. |  |
| --- | --- |
|  | 77 |

| Operation modes of the 8253 Mode 0 (Interrupt on Terminal Count) - Mode 0 is used for the generation of accurate time delay under software control. - In this mode, the counter will start counting from the initial COUNT value loaded into it, down to 0. - Counting rate is equal to the input clock frequency. - The OUT pin is set low after the Control Word is written, and counting starts one clock cycle after the COUNT programmed. - OUT remains low until the counter reaches 0, at which point OUT will be set high. - OUT will remain high until the counter is reloaded or the Control Word is written. - The Gate signal should remain active high for normal counting. - If Gate goes low counting get terminated and current count is latched till Gate pulse goes high again |  |
| --- | --- |
|  | 78 |

79

Figure 7: Illustration of Mode 0 operation.

| Operation Modes of the 8253 Mode 1 (Programmable One-Shot ) - GATE input is used as a trigger input. - OUT will be initially high. - OUT will go low on the CLK pulse following a trigger to begin the one-shot pulse, and will remain low until the Counter reaches zero. - OUT will then go high and remain high until the CLK pulse after the next trigger. - After writing the Control Word and initial count, the Counter is armed. - A trigger results in loading the Counter and setting OUT low on the next CLK pulse, thus starting the one-shot pulse. - An initial count of N will result in a one-shot pulse N CLK cycles in duration. |  |
| --- | --- |
|  | 80 |

| Operation Modes of the 8253 Mode 1 (Programmable One-Shot ) - The one-shot is retriggerable, hence OUT will remain low for N CLK pulses after any trigger. - The one-shot pulse can be repeated without rewriting the same count into the counter. - If a new count is written to the Counter during a one-shot pulse, the current one-shot is not affected unless the counter is retriggered. - In that case, the Counter is loaded with the new count and the one-shot pulse continues until the new count expires. |  |
| --- | --- |
|  | 81 |

| Figure 8: Illustration of Mode 1 operation. |  |
| --- | --- |
|  | 82 |

| Operation Modes of the 8253 Mode 2 (Rate Generator) - In this mode, the device acts as a divide-by-n counter, which is commonly used to generate a real-time clock interrupt. - Like other modes, counting process will start the next clock cycle after COUNT is sent. - OUT will then remain high until the counter reaches 1, and will go low for one clock pulse. - OUT will then go high again, and the whole process repeats itself. - The time between the high pulses depends on the preset count in the counter's register, and is calculated using the following formula: value to be loaded into counter = finput/foutput |  |
| --- | --- |
|  | 83 |

84

Figure 9: Illustration of Mode 2 operation.

| Operation Modes of the 8253 Mode 3 (Square Wave Generator) - This mode is similar to mode 2. - However, the duration of the high and low clock pulses of the output will be different from mode 2. - Suppose n is the number loaded into the counter (the COUNT message), the output will be - high for n/2 counts, and low for n/2 counts, if n is even. - high for (n+1)/2 counts, and low for (n-1)/2 counts, if n is odd. |  |
| --- | --- |
|  | 85 |

86

Figure 10: Illustration of Mode 3 operation.

| Operation Modes of the 8253 Mode 4 (Software Triggered Strobe) - After Control Word and COUNT is loaded, the output will remain high until the counter reaches zero. - The counter will then generate a low pulse for 1 clock cycle (a strobe) – after that the output will become high again. |  |
| --- | --- |
|  | 87 |

88

Figure 11: Illustration of Mode 4 operation.

| Operation Modes of the 8253 Mode 5 (Hardware Triggered Strobe) - This mode is similar to mode 4. - However, the counting process is triggered by the GATE input. - After receiving the Control Word and COUNT, the output will be set high - Once the device detects a rising edge on the GATE input, it will start counting. - When the counter reaches 0, the output will go low for one clock cycle – after that it will become high again, to repeat the cycle on the next rising edge of GATE. | . |
| --- | --- |
|  | 89 |

90

Figure 12: Illustration of Mode 5 operation.

|  | 91 |
| --- | --- |

93

94